CASE STUDY:

Operational Excellence & IIoT Integration at Plant PP-16

Facility: Solar Industries India Ltd. (Nagpur Complex)

Core Product: Superpower 90 Packaged Emulsion Explosives

Focus: Process Automation, Quality Resilience, and Safety Infrastructure

1. Executive Overview

The PP-16 manufacturing unit serves as a flagship facility for Solar Industries India Ltd., specializing in the production of Superpower 90. This unit bridges the gap between traditional chemical manufacturing and "Industry 4.0" by integrating PLC-driven automation and IIoT-enabled performance monitoring.

This case study evaluates the end-to-end production lifecycle at PP-16, highlighting the plant's ability to maintain high-specification quality ($1.20 \text{ g/cc}$ density) while ensuring zero-waste operations through innovative product diversion strategies.

2. Infrastructure & Technical Specifications

2.1 Plant Design & Capacity

The PP-16 facility is a high-volume Packaged Emulsion Explosives Unit designed for continuous, high-precision manufacturing. Unlike traditional batch-processing plants, PP-16 is engineered as a Modular Flow System, allowing for seamless transition between different cartridge diameters while maintaining strict chemical stability.

- Facility Designation: PP-16 (Strategic Manufacturing Unit).

- Production Capacity: [Insert Capacity here, e.g., 50 Metric Tons/Day]. The plant is optimized for a 24/7 operational cycle, supported by a rapid-changeover KP machine setup to minimize downtime between different product sizes.

- Technology Stack & Control Architecture:

  - Automation Layer: The heart of PP-16 is a Centralized PLC System (Siemens/Rockwell). This system manages the "Phase-Ratio Control," which is the critical synchronization between the aqueous phase and the oil phase. The PLC ensures that the mass flow of ingredients remains constant within a $0.5\%$ margin of error, preventing chemical imbalances that could affect the explosive's sensitivity.

  - Intelligent Monitoring (IIoT): The plant is embedded with a network of industrial-grade sensors that create a "Digital Twin" of the production environment.

    - Thermal Sensors: Monitor the temperature of the emulsion at the jet mixer and emulsifier to prevent premature gassing or crystallization.

    - Pressure Transducers: Track the "Back-Pressure" at the KP machine to ensure the emulsion isn't being over-compressed, which could degrade its water resistance.

    - Flow Meters: Mass flow meters monitor the precise consumption of AN (Ammonium Nitrate) against the final output of 25kg boxes.

  - Data Analytics & Decision Support: The facility utilizes a custom OEE (Overall Equipment Effectiveness) Dashboard. This platform aggregates sensor data into actionable insights, such as:

    - Availability: Live tracking of "Micro-Stops" (e.g., waiting for AN bags or box refills).

    - Performance: Comparing current machine cycles against the "Ideal Cycle Time" for specific cartridge sizes (e.g., 25mm vs. 90mm).

    - Quality: Real-time logging of "Rejected vs. Accepted" cartridges at the scanner stage to identify wear and tear in the emulsifier blades early.

2.2 Product Profile: Superpower 90

Superpower 90 is a detonator-sensitive, putty-like emulsion explosive.

- Velocity of Detonation (VOD): Minimum $3.5 \text{ km/sec}$.

- Relative Weight Strength: 118% (compared to ANFO).

- Water Resistance: Excellent (rated for 20m depth in watery holes).

3. The Production Process Flow (End-to-End)

The PP-16 plant operates a Continuous Emulsion Line, a sophisticated manufacturing method where raw materials are transformed into a high-velocity explosive through precise mechanical shearing. The workflow is divided into three critical phases:

Phase 1: Raw Material Integration & Primary Mixing

In this phase, the ingredients are prepared to form the "oxidizer-fuel" matrix. Accuracy here determines the final explosive power.

- Thermal Preparation (OB/FB Tanks): The Oil Phase (OB) and Fuel Phase (FB) are held in temperature-controlled, jacketed tanks. Pre-heating these phases is essential to lower viscosity, ensuring that when they reach the mixer, they achieve a perfect molecular bond.

- The Jet Mixing Stage: Utilizing high-velocity injection, the OB and FB are introduced into a Jet Mixer. This creates a turbulent environment where the phases begin to unite.

- Precision AN Feeding: Bulk Ammonium Nitrate (AN), provided in industrial bags, is introduced via a calibrated screw-feeder. The PLC synchronizes the feeder speed with the liquid flow rates to maintain the exact stoichiometric ratio required for Superpower 90.

Phase 2: High-Shear Emulsification & Stabilization

This is the "Chemical Heart" of the plant, where the mixture gains its explosive properties and water resistance.

- Buffer Management (Product Hopper): The primary mixture is gravity-fed into a Product Hopper. This hopper acts as a surge tank, absorbing any fluctuations in the upstream feed to ensure the Emulsifier receives a constant, uninterrupted "head" of material.

- The Emulsifier (High-Shear Mixing): The mixture enters the High-Shear Emulsifier, which uses rapidly rotating blades to break the oxidizer solution into microscopic droplets, coating each one in a thin film of fuel/oil. This process—Emulsification—is what gives the product its "putty-like" consistency and ensures it remains stable even when submerged in 20m of water.

Phase 3: Mechanical Forming & Quality Validation

The final phase focuses on physical dimensions, thermal stabilization, and rigorous quality gate-keeping.

- The KP Machine (Cartridging): The finished emulsion is pumped into the KP Machine. This specialized equipment extrudes the emulsion into plastic films, clipping them at precise intervals to create cartridges. The machine is versatile, capable of producing diameters from 25mm (for small-scale blasting) up to 90mm (for heavy mining).

- Thermal Stabilization (Cooling Tank): Freshly formed cartridges are "hot" due to the friction of emulsification. They travel through a Cooling Tank filled with temperature-regulated water. This rapid cooling "sets" the emulsion, preventing the internal chemical structure from breaking down.

- Automated Scanning & Rejection: Every cartridge passes under a High-Precision Scanner. This system checks for:

  - Weight Accuracy: Ensuring the consumer gets exactly what is promised.

  - Seal Integrity: Checking for leaks that could lead to desensitization.

  - Density Check: Confirming the $1.20 \text{ g/cc}$ target.

- Final Packaging: Cartridges that pass the scan are conveyed to the final stage, where they are mechanically packed into 25kg boxes. These boxes are then palletized and marked with batch codes for full traceability.

Quality Pivot: The "Solar Gel" Protocol

One of the most efficient aspects of the PP-16 flow is the Non-Conformance Loop. If the scanner detects a cartridge that does not meet the "Superpower 90" standard, the system does not halt production. Instead, the material is diverted, safely reprocessed, and converted into Solar Gel. This ensures that the plant maintains a High-Efficiency Ratio and reduces the cost of raw material waste to nearly zero.

E FOR FLOWCHART: "Process Flow Diagram of PP-16 Emulsion Line"]

[SPACE FOR IMAGE: "KP Machine and Cartridging Area"]

4. Digital Transformation: IIoT & OEE Monitoring

The "intelligence" of PP-16 lies in its ability to convert physical sensor data into operational decisions.

4.1 Real-time Parameter Tracking

The plant monitors three critical vectors:

- Temperature: Essential for chemical sensitization; deviations trigger an automatic PLC shutdown.

- Pressure: Monitored at the emulsifier and KP machine to prevent blockages or friction-related risks.

- Flow Rates: Ensures the density of Superpower 90 remains within the $1.20 \pm 0.05 \text{ g/cc}$ range.

4.2 OEE Calculation

By tracking "Availability, Performance, and Quality," PP-16 identifies hidden losses:

- Availability: Tracking downtime during AN bag changes.

- Performance: Measuring KP machine speed against its theoretical maximum.

- Quality: Tracking the ratio of "First Pass" Superpower 90 against rejected batches.

5. Quality Assurance & Zero-Waste Strategy

PP-16 employs a unique "Diversionary Quality Protocol": If a batch fails the scanner or density check:

- It is immediately Rejected from the Superpower 90 line.

- It is Reprocessed into Solar Gel (a chemically gassed product for open-cut mines).

- Outcome: 0% chemical waste and 100% resource utilization.

6. Safety Protocols & Compliance

Operating under PESO (Petroleum and Explosives Safety Organisation) guidelines, PP-16 prioritizes the "Human Element."

- Static Discharge: All equipment is fitted with static dischargers to ground any frictional electricity.

- Blast Protection: Physical Safety Walls (Blast Bunds) isolate the emulsification zone.

- PPE Standards: Mandatory use of antistatic shoes, industrial helmets, safety goggles, and chemical-resistant masks.

[SPACE FOR IMAGE: "Safety Infrastructure & Safety Wall Layout"]

[SPACE FOR IMAGE: "Final Packaged 25kg Boxes of Superpower 90"]

. Key Outcomes & Findings

The case study of PP-16 reveals three major outcomes:

Outcome 1: Enhanced Product Consistency

Through the use of automated AN feeders and PLC jet mixers, PP-16 has achieved a 98% consistency rating in product density, significantly reducing the "scatter" in VOD (Velocity of Detonation) during field use.

Outcome 2: Operational Efficiency (OEE) Gains

The implementation of IIoT monitoring allowed the plant to identify that 15% of downtime was caused by manual AN bag feeding.

- Result: Potential for future upgrade to bulk-silo feeding to increase OEE by an estimated 10-12%.

Outcome 3: Superior Safety Record

By combining physical safety walls with electronic static dischargers, PP-16 maintains a Zero-Incident Rating. The automated scanning system also removes the need for human operators to manually handle "hot" or freshly formed cartridges.

Outcome 4: Economic Sustainability

The conversion of rejected Superpower 90 into Solar Gel has saved the plant an estimated 5-8% in raw material costs annually that would have otherwise been lost to waste disposal.